# EECE321 Word Scrambler Descrambler Project Report

American University of Beirut

Dr. Yasser Mohanna

Carla Jaffal | Michel Katrib

# 1. Introduction

The goal of this project is to write a RISC-V assembly program that scrambles and descrambles a text message. The program asks the user to enter a message of 9 to 15 words and then asks for a numeric seed. The program rearranges the order of the words using random indices generated from the seed, prints the scrambled sentence, then reverses the process and prints the descrambled output.

The program treats the message as a list of words instead of a list of individual characters. The characters inside each word are not changed. Only the positions of the words are exchanged. This makes the solution efficient because the program swaps word addresses rather than copying complete words.

# 2. Design

The assembly code is organized into five main sections: taking input, validating the seed and word count, splitting the string into words, scrambling the word order, and descrambling the result. The data section reserves 200 bytes for the input message, 60 bytes for word_pointers, and 60 bytes for swap_indices. The 60-byte arrays are enough because the maximum number of words is 15 and each stored address or index is 4 bytes.

The most important design choice is the use of word_pointers. After the input is validated, the program stores the address of every word in this array. Later, scrambling only swaps entries inside word_pointers; the original characters remain in the same memory locations. This keeps the algorithm simple and avoids unnecessary character-by-character copying.

## 3, Input Validation and Splitting

The program reads the message using syscall 8 and reads the seed using syscall 5. The seed is stored in s0, then checked with bgtz. If the seed is zero or negative, the program prints the invalid seed message and exits.

Next, the program counts the words in the message. Register t0 is used as a pointer to scan the string, t2 stores the word count, and t3 is used as a flag to determine whether the scan is inside or outside a word. When a non-space character is found while the flag is zero, a new word has started and the count is incremented. The scan stops at the null terminator or newline character. The final count must be between 9 and 15; otherwise, the program prints the invalid message error and exits.

After the word count is accepted, the program scans the string again. Each time a new word begins, its address is stored in word_pointers. Each space is replaced by \0, and the newline is also replaced by \0. As a result, every word becomes a separate null-terminated string that can be printed directly with the print-string syscall.

## 4. Scrambler

The program uses a custom random number generator named random_num_gen. The seed is copied from s0 into x9, which acts as the generator state. The routine works on a 16-bit value: it creates a feedback bit using XOR operations on shifted versions of the state, shifts the state left, masks it with 0xffff, and inserts the feedback bit as the new least significant bit.

During scrambling, t0 is the loop index i. For each word position, the program calls random_num_gen, then computes j = x9 % word_count using rem. This guarantees that j is a valid index. The value of j is stored in swap_indices[i], then the program swaps word_pointers[i] with word_pointers[j].

When all swaps are complete, the program prints Scrambled Output: and loops through word_pointers, printing each word in its new order followed by a space.

## 5. Descrambler

The descrambling step works by reversing the same swaps. Since swapping two values is its own inverse, the program can restore the original order by applying the stored swaps backward.

The loop starts at word_count - 1 and decrements until the index becomes negative. At each step, the program loads j = swap_indices[i] and swaps word_pointers[i] with word_pointers[j] again. Because this is done from the last scrambling swap back to the first one, the pointer array returns to its original order. The program then prints Descrambled Output: and prints the words again.

## 6. Registers and Syscalls

The seed is kept in s0, while x9 is used as the current random generator state. The word count is kept in t2 so it can be reused during splitting, scrambling, printing, and descrambling. As Dr Yasse explained, temporary registers such as t0, t1, t3, t4, t5, and t6 are used for character scanning, branch checks, array offsets, and pointer calculations. Registers a0 and a7 are used for RARS syscalls.

The program uses syscall 4 to print strings, syscall 8 to read the input message, syscall 5 to read the integer seed, syscall 1 to print the word count, syscall 11 to print spaces and newlines, and syscall 10 to exit.

## 7. Program Structure

The code is written with comments before the major sections. The main parts:

- random_num_gen: generates the next random state.

- word_Count: counts the number of words in the input.

- split_string: stores word starting addresses and separates the words.

- loop_scrambling: generates random indices and swaps word pointers.

- loop_descramble: reverses the saved swaps.

- print_scrambled and print_descrambled: print the two final outputs.

Each section has one main responsibility. Michel and Carla wrote enough comments to explain the purpose of the important registers and memory arrays.

# 8. Results

The program was tested with valid and invalid inputs. For valid messages, it asked for the seed, printed the correct word count, produced a scrambled word order, and then restored the original message in the descrambled output.

# 9. Conclusion

This project implements a complete RISC-V assembly text scrambler and descrambler. The program validates the user input, separates the message into words, scrambles the word order by swapping pointers, and restores the original order by applying the saved swaps in reverse. Overall, the project demonstrates byte-level string processing, pointer arrays, memory access, conditional branching, loop control, custom random number generation, and RARS input/output system calls.